ANSYS FLUENT WORKSHOP: MIXING TEE

In this workshop, the task was to create and simulate a flow simulation on a mixing tee structure, where low air speed streams would be mixed, where the first inlet would be of 25°C and the second inlet would be that of 55°C. In doing this simulation, precious money and man-hour would be saved in recreating the experiment, if not for the simulation provided by a computational fluid dynamics (CFD) software.

The first step is to create our mixing tee first in ANSYS SpaceClaim CAD software, where the dimensions and constraints were already given (See Fig 1.1), a mesh would then be created based on the shape in ANSYS Meshing Software and adding Named Section for ANSYS Fluent to recognize the position of the inlets and outlet. (See Fig 1.2)

By which a check of the skewness and the orthogonal quality of each mesh node must be performed, so that an optimal and accurate simulation could be performed. Where the skewness should be between 0 to 0.25 is the optimum level, while the orthogonal quality requires 0.95 to 1.00 for the best possible results. A link between the cells would then be established, so that ANSYS Fluent itself can begin its work.

In ANSYS Fluent, a check in the mesh quality would need to be done again to confirm that it is acceptable for the calculations, the necessary settings would then need to be properly calibrated and set; checking that the Energy box is ticked under the Physics tab – since heat transfer is being delt with in this instance, and that the k-Omega SST turbulence model are selected. (See Fig 1.3)

Boundary conditions are then now selected for each respective named section, with the appropriate parameters (Air, 2 m/s, 55°C for Inlet X and Air, 5 m/s, 25°C for Inlet Y). A new Surface Report is then defined as Uniformity Index – Area Weighted, with static temperature ticked and outlet named section is selected, we can then now initialize and calculate. (See Fig 1.4)

We would then be presented with two graphs, one being residuals plots and another being monitor plots, to get a visual result – we must create fluxes report, in which we can display them either as a contour or a vector. (See Fig 1.5)